Metadata for Map WaBeHuCa_grav_2014_all_svdu2_mil_v2

Map name:  WaBeHuCa_grav_2014_all_svdu2_mil_v2, available in both a geo-tif (*.tif)  and a Geosoft map (*.map) format.

Database author(s):  V. W. Chandler

Publishing organization: (Minnesota Geological Survey)

Date of publication:  2018

Date of map:  Map created March, 2015.

Updates: None planned.

Map key words:  Gravity, Bouguer anomaly, second vertical derivative, upward continued, Wadena, Becker, Hubbard, Cass, Minnesota

Horizontal accuracy (appropriate to final map scale): Optimum is +/- 20 meters

Coordinate system:  UTM, NAD83, zone 15N

Map area: Wadena, Becker, Hubbard, and Cass County Geologic Atlas (WaBeHuCa CGA)  area:

West bounding coordinate	218,000

East bounding coordinate	455,000

North bounding coordinate	5,270,000

South bounding coordinate	5,105,000

Grid files associated with map:   Map based on Geosoft grid WaBeHuCa_grav_2014_all_svdu2_mil_v2.grd

Description of map:  Presents the second vertical derivative of the Bouguer gravity anomaly of the WaBeHuCa CGA area, using a color, shaded relief format. Data are processed so that perturbations, or “anomalies”, in this field reflect spatial variations in density in the underlying substrate. The derivative operation enhances the signature of sources that are either at or near the bedrock surface. Maps of this kind are useful in mapping and interpreting Precambrian rocks in the subsurface

Map scale:  1:100,000.

Map units:, Milligal (mGal)/km.^2

List of features, accuracy and scales of fieldwork and compilation: Gravity data were acquired at individual ground stations that were spaced about one-mile along drivable roads and trails. Location was usually based on 1:24,000 topographic maps, and is optimally accurate to +/- 20 meters.  Elevations at stations located at spot elevations on topographic maps are probably accurate to 1 foot; elsewhere the elevations are accurate to about 3 feet. Most Bouguer gravity anomaly values at stations are accurate to within 01.-0.5 milligal. The error of the derivative data is difficult to assess, but is likely to be less than 0.01 mGal/km.^2 in most low-gradient areas.

Summary of procedures for making map:  The principal fact gravity data of this project were derived from a state-wide database. All data have been corrected for temporal variations and converted to milligals, followed by corrections for  latitude- and elevation-related effects. Corrections were made for earth curvature, but because of the generally low topographic relief across most of Minnesota, no terrain corrections have been made. The resulting Bouguer anomaly data were gridded state-wide at a 1,000 meter interval using minimum curvature interpolation, thereby producing grid Minngrav_2014_all.grd. These data were enhanced by computing the second vertical derivative, with slight smoothing for noise by upward continuing the data 2 km. In order to convert the smoothed derivative data into larger, more manageable units (from mGals/m^2 to mGals/km^2) the grid values were multiplied by 1,000,000, thereby producing the grid Minngrav_2014_all_svdu2_mil.grd. The grid . WaBeHuCa_grav_2014_all_svdu2_mil_v2.grd was windowed from the state-wide grid using the bounding coordinates cited above.. All grid-related operations were conducted using the Geosoft-Montaj software system. More information on the state-wide gravity program and available data is available through the MGS website at https://www.mngs.umn.edu/.

Lineage:  The gravity data were acquired during 1955-2006 as part of a state-wide program by the Minnesota Geological Survey (MGS), with support mainly from the Minnesota Legislature through the State Special appropriation, and the Legislative Commission on Minnesota Resources. The gravity data were upgraded in 2011 by the MGS, through support from the Minerals Coordinating Committee of the Minnesota Legislature. Upgrades included recovering archival information that was omitted from the earlier compilations (date, field number, instrumentation, etc.), and using GIS technology to reduce locational errors in the older data.

Attributes  of grid  WaBeHuCa_grav_2014_all_svdu2_mil_v2.grd

Cell Size = 1,000 meters Data Type: Floating Point

Number of E-W Rows = 166

Number of N-S Columns = 238

Xmin = 218,000

Xmax = 455,000

Ymin = 5,105,000.

Ymax = 5,270,000

Z units = milligal

Minimum Z = -1.386 mGal/km.^2

Maximum Z = 1.476 mGal/km.^2

Mean Z = -0.005 mGal/km.^2

Std. Dev.Z = 0.278 mGal/km.^2

Off-Site Z values = none

Parameters:

The GRD file is in GEOSOFT grid format which consists of a 512 byte header (combination of 4 byte signed integers and 8 byte real values) that contains parameters regarding data storage, geographic information and data scaling, followed by the grid data (4 byte real).  The accompanying gi file that contains projection information and any associated metadata for the grid.  Grid nodes are cell-centered and the grid is ordered by rows, starting from bottom-left (southwest origin).

Web access for gravity data:  https://www.mngs.umn.edu/

Contact for data: Rich Lively at MGS; Ph. 612-626-3103, or lively@umn.edu

Contact at MGS to obtain grids or line data on DVD: Val Chandler at MGS, Ph. 612-626-4976, or chand004@umn.edu ; or contact MGS, Ph 612-626-2969 .