Metadata for Map WaBeHuCa_micro10_micro_fill_rtp_fvd_330rj_v2

Map name:  WaBeHuCa_mag10_micro_fill_rtp_fvd_330rj__v2, available in both a geo-tif (*.tif)  and a Geosoft map (*.map) format.

Map author(s):  V. W. Chandler

Publishing organization: (Minnesota Geological Survey)

Date of publication:  2018

Date of map:  Created in March, 2015

Map key words:  Aeromagnetic data, micro-leveled, reduced to pole, first vertical derivative, strike-reject, Minnesota, Wadena, Becker, Hubbard, Cass

Horizontal accuracy (appropriate to final map scale): Optimum is +/- 50 meters

Coordinate system:  UTM, NAD83, zone 15N

Map area:  Wadena, Becker, Hubbard, and Cass County Geologic Atlas (WaBeHuCa CGA)  area:

West Bounding Coordinate	218,000

East Bounding Coordinate	455,000

North Bounding Coordinate	5,270,000

South Bounding Coordinate	5,105,000

Grid files associated with map:   Map based on Geosoft grid WaBeHuCa_mag10_micro_fill_rtp_fvd_330rj_v2 (*.grd, *.grd.gi)

Description of map:  Presents the first vertical derivative of the total field magnetic intensity anomaly of the geomagnetic field, which has been reduced to vertical polarization (reduced to pole), and subsequently strike-reject filtered along a 330 degree azimuth. The map uses a gray-scale format. Data are processed so that perturbations, or “anomalies”, in this field reflect physical and spatial variations in magnetic minerals in the underlying substrate. The reduction to pole procedure places anomalies more directly above their sources, and the derivative operation enhances the effect of sources that are either at or near the bedrock surface.  The strike-reject filtering was applied to attenuate the effect of  northwest-striking dikes, which locally interfere with mapping the Archean and Paleoproterozoic features that the dikes cross-cut. Maps of this kind are useful in mapping and interpreting Precambrian rocks in the subsurface. The effects of cultural sources may be locally present.

Map scale:  1:100,000

Map units:  NanoTesla (nT)/meter.

List features and accuracy as shown on map, including scales of fieldwork and compilation:  Grid based on data from north-south flight lines that were photo-located 2-5 km. along line at an optimal precision of 50 meters, although locational error may be slightly more between spotted points.  Data were originally located and compiled at a 1:24,000 scale.  The values in the original total intensity grid are probably accurate to +/- 5 nanoTesla or less in most areas.

Summary of procedures for making map:  Total magnetic intensity values are based on magnetic readings taken along north-south flight lines that were usually spaced 400 meters apart and 150 meters above terrain.  Following photo-based flight path recovery, the magnetic data were corrected for temporal variations and the regional effects of the earth’s magnetic field. . The corrected data were gridded at a 100 meter interval using minimum curvature interpolation.  These data were merged into a state-wide grid, which used a state-wide datum of 150 meters above surface for compilation. Grid WaBeHuCa_mag10_micro_fill_v2.grd was windowed from state-wide grid minmag10-micro_fill.grd, using the bounding coordinates cited above. The windowed grid was reduced to pole assuming a magnetic declination of 5.1 degrees and magnetic inclination of 74,4 degrees, thereby producing grid WaBeHuCa_mag10_micro_fill_rtp_v2.grd. The reduced to pole grid was enhanced by computing the first vertical derivative, thereby producing grid WaBeHuCa_mag10_micro_fill_rtp_fvd_v2.grd. This grid in return was strike-reject filtered along a 330 degree azimuth, thereby producing grid WaBeHuCa_mag10_micro_fill_rtp_fvd_330rj_v2.grd All grid-related operations were conducted using the Geosoft-Montaj software system. More information on the state-wide aeromagneic program and available data is available through the MGS website at https://www.mngs.umn.edu/.

Lineage:  The magnetic data were acquired in 1981-1987 as a part of a state-wide aeromagnetic survey program that was conducted by the MGS with support from the Minnesota Legislature as recommended by the Legislative Commission on Minnesota Resources (now LCCMR).  The magnetic data were upgraded in 2005-2007 through a program funded by the Minnesota Legislature as recommended by the Minerals Coordinating Committee.  Tasks of this latter program included recovering missing data, correcting line data for minor leveling errors (temporal variations), and creating a new state-wide grid with improved resolution.  More details on the aeromagnetic data upgrade are available at https://www.mngs.umn.edu/.

Attributes of Grid WaBeHuCa_mag10_micro_fill_rtp_fvd_330rj__v2.grd:

Cell Size = 100 meters Data Type: Floating Point

Number of E-W Rows = 1651

Number of N-S Columns = 2371

Xmin = 218,000

Xmax = 455,000

Ymin = 5,105,000.

Ymax = 5,270,000

Z units = nanoTesla (nT)/m.

Minimum Z = -9.805 nT/m.

Maximum Z = 30.394 nT/m.

Mean Z = -0.004 nT/m.

Std. Dev.Z = 0.445 nT/m.

Off-Site Z values = none

Parameters:

The GRD file is in GEOSOFT grid format which consists of a 512 byte header (combination of 4 byte signed integers and 8 byte real values) that contains parameters regarding data storage, geographic information and data scaling, followed by the grid data (4 byte real).  The accompanying gi file that contains projection information and any associated metadata for the grid.  Grid nodes are cell-centered and the grid is ordered by rows, starting from bottom-left (southwest origin).

Web access for aeromagnetic data:  https://www.mngs.umn.edu/

Contact for data: Rich Lively at MGS; Ph. 612-626-3103, or lively@umn.edu

Contact at MGS to obtain grids or line data on DVD: Val Chandler at MGS, Ph. 612-626-4976, or chand004@umn.edu ; or contact MGS Map Sales, Ph 612-626-2969.